| School/Institute/Directorate: | Location: | Supervisor | Date: | Review Date: |
| --- | --- | --- | --- | --- |
| EPS | Energy Lab, Dubai | Dr Rehan Ahmed | 2023-09-11 | Whenever there is a change in SOP and risk involved. Last update: 2023-09-11 |

| Risk Assessment Title: Ultrasonic Bath & related | Ref No: 2023-2024 |
| --- | --- |

| Risk Rating Matrix (RR) | Likelihood(L) |  |  |
| --- | --- | --- | --- |
| Severity (S) | Certain or near certain to occur (High) | Reasonably likely to occur (Medium) | Unlikely to Occur (Low) |
| Fatality, major injury or illness causing long term disability (High) | HIGH(H) | HIGH(H) | MEDIUM(M) |
| Injury or illness causing short term disability (Medium) | HIGH(H) | MEDIUM(M) | LOW(L) |
| Other Injury or Illness (Low) | MEDIUM(M) | LOW(L) | LOW(L) |

Description:

Background: In the creation and analysis of metallographic samples, it is necessary to clean said samples between activities. The ultrasonic bath is a benchtop device that uses ultrasound energy within liquids to remove residual material from the surface of samples. Acetone is often used due to its superior cleaning properties and its quick evaporation, allowing for faster cleaning, and drying of samples before weighing. Acetone can be found in the Flammable cupboard (further details below on use) if not available at ultrasonic bath station. Personal Protective Equipment (PPE) Wear gloves, safety glasses, and lab coat while using chemicals (eg. Acetone). Enclosed shoes must be worn while using the machine. Procedural Controls Avoid using finger Consult Material Safety Datasheet (MSDS) before using machine and during training. Users should operate the bath in accordance with training to avoid damage to the instrument. Avoid accessing the rear of instrument. Fill bath with clean/distilled water only. Low stocks of consumable items (gloves, paper towels, etc) should be reported to Supervisor or Lab Manager.

| What are the Hazards? | Who might be harmed? | Uncontrolled Risk Rating | Control measures (What are you already doing?) | Controlled Risk Rating | Responsible Person(s) |
| --- | --- | --- | --- | --- | --- |
| Fumes | Equipment User | Low | Use of well-ventilated space. | Low | Lab Manager |
| Noise | Equipment User Lab User | Low | Avoid using ultrasonic cleaner while using nearby equipment. Use of ear protection. | Low | Equipment User Supervisor |
| Electric Shock | Equipment User Lab User | High | Use of waterproof electrical socket for electrical connection. Avoid touch of electrical source and connections using wet hands. Keep hands dry whenever touch the electrical source. Switch off the ultrasonic cleaner when not in use. PAT Test equipment yearly. | Low | Lab Manager |
| Using flammable solvents in ultrasonic cleaner | Equipment User Lab User | Medium | The ultrasonic bath is not to be filled with flammable solvents (eg. IPA, acetone) as flammable solvents pose a serious explosion hazard. Small parts are to be cleaned inside beakers that are partially immersed in water – the waves pass through the beakers wall. | Low | Equipment User Supervisor |
| Storage of flammable chemicals | Equipment User | High | Ensure that large quantities of acetone are stored in the flammable liquids cupboard in the Chemical lab and that only small quantities are utilized near the station. In the event of a spill, avoid use of nearby equipment without removing spilled liquid. | Low | Equipment User |
| Cuts from broken glass | Equipment User | Low | Avoid using chipped or broken glassware. Use nitrile gloves while handling acetone. Avoid touching broken glass with bare hands. | Low | Equipment User |

Additional Work Practice Information: Avoid use of beakers on bottom of ultrasonic bath tank; the vibration induced may wear the tank surface leading to leaks and damage. If Equipment Users are sensitive to noise or poor quality air, avoid use of uncovered operation of ultrasonic cleaner and use N95 masks. Facilities Management and Housekeeping usually do not touch equipment. Keep space around ultrasonic equipment clean to avoid interference. Avoid use of ultrasonic cleaner when classes occur in the lab. Although inconvenient, avoiding noise disturbance is a courtesy worth extending to other lab users.

| Name |  | Position | Email | Signature |
| --- | --- | --- | --- | --- |
| Author | Vishakh Pradeep Kumar | MSc Student; Research Assistant | v.kumar@hw.ac.uk | Vishakh Pradeep Kumar |
| Authoriser | Dr Rehan Ahmed | Supervisor | r.ahmed@hw.ac.uk |  |
| Verifier |  |  |  |  |

Declaration

I confirm that I have read the Risk Assessment, I understand the hazards, and risks involved and will follow all the control measures stated. If I have any queries or concerns, I will contact the above.

| Name | Organisation | Staff/Student/Visitor | Signature | Date |
| --- | --- | --- | --- | --- |
| Vishakh Pradeep Kumar | EPS | Student | Vishakh Pradeep Kumar | 2023-09-11 |